Packaging specification: 10

Non-Protective Packaging for Plates with Overlap and Straps

Code 10: Non-protective packaging for plates – export

(Available on enquiry only)

- Labels - centre, opposite edges - top surface

- Die stamp marks

- Stencil mark

- Harbour colour marks

- Straps

- Wood

- Bundle

- Lift

Specifications

|  | Plate mill |
| --- | --- |
| Local | L  15 000mm |
| Export | L  12 000mm |
| Rail | W  2 400mm |
| Road | W  3 200mm |
| Wall Height | 350mm |
| Lift mass | Bundles: 13T export with 1 - 5 T bundles (overlaps) |

Additional notes:

i.	Bundle mass = packing mass

ii.  Lift mass  max. handling mass

Strapping

| Plate Length (mm) | No of straps |
| --- | --- |
| L < 6000mm | 3 |
| 6000  L< 8000mm | 4 |
| L ≥ 8000mm | 5 |

## Corrosion considerations

- Packaging classified as non-protective provides no protection against corrosion, accordingly the material could be susceptible to corrosion under wet and humid conditions.

- The plastic is not protected and could tear during normal handling operations. The likelihood of premature surface deterioration will be enlarged.

Corrosion guarantee

| Description | Product code | SPE | * Allowed claim period per surface treatment |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | None | Passivated | Dos oil | Rust preventative oil |
| Steel Plates | MP, MPSQ, MPLQ, MPG | 110/111 | 0 | n/a | n/a | n/a |

* Days after receipt of material.

n/a- not available.

Note:

Depending on the prevailing weather conditions and precautions taken during transit, the material may corrode as no protection is provided by this packaging method.